|  | CUSTOM ORDER GUIDE Laser etching |
| --- | --- |

| EXAMPLE TIP SIZE 10mm × 25mm | APPROX. ETCH AREA ~18 × 6mm | FINISH Frosted matte on clear | COLOR Frosted look only |
| --- | --- | --- | --- |

01  Logo shape decides everything

RipTips are small and curved. The visible face of a 10mm cylinder is only ~6-7mm wide before the curve takes over. Rectangular logos use the full length and stay sharp. Square and circular logos shrink to fit, and detail disappears.

| WHAT TO SEND • Vector files only: .ai, .eps, .svg, or .pdf • Single color, black on transparent • All text converted to outlines • No JPEG or PNG (rasters can’t etch) | THREE RULES • Strip it down. Drop the tagline and est. date • Test at actual size: print at 18mm wide • Bold beats delicate. No lines under 0.5pt | HOW IT WORKS • Send files to info@theriptip.com • Best on sizes 10mm to 12mm, where the etch area is largest. We don’t recommend etching on 7mm tips • Production: 2 to 3 weeks on in-stock tips. 100 tip minimum, can be split across sizes |
| --- | --- | --- |

| Can I get my logo in color? Not with etching. Decals support 2-3 colors. | Can you redraw my logo? No, please send finished vector files ready to etch. | What if it doesn’t fit? We’ll flag it before production and ask for alternatives. |
| --- | --- | --- |

theriptip.com   /   info@theriptip.com   /   Made in USA